WIFI CONFIGURATION USING NRF CONNECT MOBILE APP:

Download and Install the NRF connect app from the google play store

Open the app

press the scan button and the RCCT_LED_BRD will show up in the scanners tab . connect to the RCCT_LED_BRD

Click on the third characheteristic (write charachteristic)

Change the type of value to text

Now paste the wifi creds JSON which goes as:

{"ssid": "your_ssid", "password": "your_password"}

after entering and sending the JSON it will look something like this:


































inorder to verify if the JSON is sent you can click on the read characteristic arrow and it will show the device connectivity status